电子科技大学2022-2023学年第2学期期末考试A卷

考试科目： 数字逻辑设计及应用 考试形式： 闭卷 考试日期：  2023 年 6 月 27 日

本试卷由  六  部分构成，共  8  页。考试时长： 120 分钟

成绩构成比例：平时成绩   60  %，期末成绩   40  %   注：禁止使用计算器及英文辞典

| 题号 | 一 | 二 | 三 | 四 | 五 | 六 | 七 | 八 | 合计 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 得分 |  |  |  |  |  |  |  |  |  |

得 分

(Please refer to the appendix for the function description of 74 chips)

I.  Please fill out the correct answers in the brackets “(    )”.

( 2’×15 = 30’ )

1. (0001 1001 0101)8421BCD = (   1100 0011  )2

2. For an odd-parity transceiver system, if the information bits sent are 10101010，then the parity bit should be (  1  ).

3. The function of DAC is to convert (  digital  ) inputs proportionally into analog outputs.

4. There are ( 2N-N ) invalid states for a N-bit Ring counter.

*5. The three output states of the tristate gate are: logic "1", logic "0", and (     Hiz      ).

*6. To design a 3-input majority judger using AND-OR structure (output is “1” when the input is majority),  ( 3 ) AND gates are required at least.

7. The FSM which its output depends not only on its states but also on the input is called ( Mealy ) machine.

*8. A NAND gate in positive logic is called ( NOR ) gate in negative logic.

9. If a RAM’s capacity is 16K words × 8 bits, the address inputs should be (  14  ) bits. We need (  8  ) chips of 16K  8 bits RAM to form a 32K  32 bits RAM.

10. If the minimum incremental output voltage of an 8-bit unipolar DAC is 0.02V, then the output voltage is (   2.72    ) V for the DAC when the input is 10001000.

11. A FSM with 130 states requires (    8   ) flip-flops at least.

*12. The implementation of 3-input CMOS NOR gate requires (      ) MOS transistors.

*13. The input waveforms A, B, C, and output waveforms F of a combination logic circuit are shown in Fig. 1. The minterm list for this circuit are F( 	  ).

Fig. 1

14. The output of the circuit shown in Fig. 2 is F(  0,1,2,4  ).

Fig. 2

得 分

II. Choose the correct answer and fill in the brackets. ( 2’×10 = 20’ )

1. A sequential circuit is shown in Fig. 3 with initial states Q2Q1 = 00. Which of the following is NOT the output serial sequence from Q1?	 ( 	C  )

Fig. 3

A. 0011									B. 1001

C. 1101									D. 0110

2. If a Johnson counter with 12 valid states is constructed by shift register, (   B   ) 74x194 shift register(s) should be used.

A. 1										B. 2

C. 3										D. 4

3. Let [A]two’s complement=10010011. Which of the following is true?  (   C    )

A. [-A]one’s complement = 01101100   			B. [A]one’s complement= 10010100

C. [-A]sign-magnitude= 01101101   				D. [A]sign-magnitude= 00010011

4. How many invalid states are there in the circuit shown in Fig. 4?  (   D    ).

A. 10									B. 11

C. 12									D. 13

Fig. 4

5. There is no static hazard in expression (    B    ).

A. F = B’C’+AC+A’B             			B. F = B’C’+AC+A’B+BC+AB’+A’C’

C. F = A’C’+BC+AB’+A’B         			D. F = A’C’+BC+AB’

*6. A gate that can implement wired AND function is called (      ) gate.

A. open-drain     						B. OR

C. XOR		    						D. AND

7. The minimum sum-of-products of  is (   A   ).

A．W’X + Y’Z + WX’Y’           			B. W’X + Y’Z + WX’Z’ + XZ

C．W’XY + Y’Z + WX’Z’                 	D. W’XZ’ + Y’X’Z + WX’Z’ + XZ

*8. If we want to use an XNOR gate as an Inverter (NOT gate), then the inputs of A and B should be connected as (     ) .

A. one of the A and B connect to ‘1’      		B. one of the A and B connect to ‘0’

C. A and B connect together            		D. can’t realize

9. The output of the following circuit shown in Fig. 5 is F = (A+B)’, when C1 and C2 are equal to (   D   ).

A．0，0                                 B．0，1

C．1，0                                 D．1，1

Fig. 5

10. The output signal of (   A    ) circuit is 1-out-of-N code.

A. binary decoder   						B. binary encoder

C. seven-segment decoder   				D. decade counter

得 分

III.  A Verilog module for a 74x163 4-bit binary counter as shown below. Please fill in the missing code on the underlines to complete the module. 	[5’]

评分标准：每空正确得1分

module M74x163(CLK, CLR_L, LD_L, ENP, ENT, D, Q, RCO);

input CLK, CLR_L, LD_L, ENP, ENT;

input [3:0] D;

output [3:0] Q;

output   RCO;

reg [3:0] Q;

reg RCO;

always @ (posedge CLK)

if (CLR_L == 0)								Q <= 4’b0;

else if (LD_L == 0)								Q <=    D    ;

else if ((ENT == 1) && (    ENP == 1    )) 	Q <= Q + 1;

else											Q <=    Q    ;

always @ (Q or ENT)

if ((   ENT == 1   ) && ( Q == 4’d15 ))		RCO = 1;

else											RCO = 0;

endmodule

得 分

IV.  Analyze the combinational logic circuit as shown below. 	[10’]

1. Write out the minterm list of outputs G2B2,B1,B0, G1B2,B1,B0, G0B2,B1,B0.	 (3’)

2. Write out the minimal sum-of-products expression of each output.   (6’)

3. Indicate the function of the circuit.	 (1’)

[Solution]

1. Minterm list （每个表达式正确得1分，共3分）

G2=B2,B1,B04,5,6,7

G1=B2,B1,B02,3,4,5

G0=B2,B1,B01,2,5,6

2. Minimal sum（每个表达式正确得2分，共6分）

G2=B2

G1=B2'B1+B2B1'

G0=B1'B0+B1B0'

3. 电路功能为将输入三位的二进制编码转换为格雷码。（正确得1分）

得 分

V.  Design a clocked synchronous state machine with the state diagram shown as below by using three D flip-flops. The state values Q2Q1Q0 of A~H are 000, 001, 010, 011, 100, 101, 110, 111 respectively.		[17’]

1. Fill in the Karnaugh maps, write out the minimal sum-of-products expressions for the excitation equations.  (12’)

2. Write out the output equation.  (3’)

3. Indicate the logic function of the state machine.  (2’)

[Solution]

1. （卡诺图每个正确得2分，表达式每个正确得2分，共12分）

D0=Q1'Q0'+Q2'Q0'+Q2Q1Q0

D1=Q1Q0'+Q2Q1+Q2'Q1'Q0

D2=Q2'Q1Q0+Q2Q1'Q0'

2. Z=Q2Q1'Q0	（正确得3分）

3. 带自启动功能的模6二进制加法计数器。 （正确得2分）

得分

VI.  Analyze the sequential logic circuit as shown below.  [18’]

1. Write out the minterm list of the output Z QB,QC,QD .  (3’)

2. Complete the transition/output table. 	(12’)

3. Assume the initial state QBQCQD is 001, write out the output sequence of Z in a cycle.  (3’)

[Solution]

1. Z=QB,QC,QD(0,2,3,4,5)		（正确得3分）

2. （转移输出表的新状态每空正确得1分，输出每空正确得0.5分，共12分）

Transition / output table

| QBQCQD | QB*QC*QD* | Z |
| --- | --- | --- |
| 000 | 001 | 1 |
| 001 | 010 | 0 |
| 010 | 101 | 1 |
| 011 | 111 | 1 |
| 100 | 001 | 1 |
| 101 | 011 | 1 |
| 110 | 100 | 0 |
| 111 | 110 | 0 |

3. Output sequence is 0111001.	（正确得3分）

Appendix